Python Code Snippet: Control Table Address Constants for XM540-W270-R

#Control Table Address Constants for XM540-W270-T/R

# Motor Identification

ADDR_ID = 7  # Identifies the motor on the communication bus

# Operating Mode

ADDR_OPERATING_MODE = 11  # Sets the operating mode of the motor

# Safety Limits

ADDR_TEMPERATURE_LIMIT = 31  # Maximum allowable temperature before shutdown

ADDR_CURRENT_LIMIT = 38      # Maximum allowable current

ADDR_MAX_POSITION_LIMIT = 48  # Maximum position limit in pulses

ADDR_MIN_POSITION_LIMIT = 52  # Minimum position limit in pulses

ADDR_SHUTDOWN = 63           # Conditions to shut down the motor

# Torque Control

ADDR_TORQUE_ENABLE = 64      # Enables or disables motor torque

# Status Feedback

ADDR_STATUS_RETURN_LEVEL = 68  # Configures the return level of status packets

# Control Gains

ADDR_POSITION_P_GAIN = 84    # Proportional gain for position control

# Motion Profiles

ADDR_PROFILE_ACCELERATION = 108  # Acceleration profile for position control

ADDR_PROFILE_VELOCITY = 112      # Velocity profile for position control

# Position Control

ADDR_GOAL_POSITION = 116     # Target position in pulses

ADDR_PRESENT_POSITION = 132  # Current position in pulses

# Current Feedback

ADDR_PRESENT_CURRENT = 126   # Current electrical load on the motor

# Temperature Monitoring

ADDR_PRESENT_TEMPERATURE = 146  # Current internal temperature of the motor