### Recommendation for Wrist Rehab Robots

### E-manual of Dynamixel XM540-W270-R: https://emanual.robotis.com/docs/en/dxl/x/xm540-w270/#position-pid-gain80-82-84

For the specific application of wrist extension and flexion rehabilitation, the best mode depends on the task:

- Current-based Position Control Mode (Best Overall):

  - Combines position accuracy and adaptive force.

  - Allows safe, smooth, and progressive rehabilitation exercises.

- Position Control Mode (0~360° or Extended Multi-turn):

  - Good for fixed ROM exercises when torque adaptability isn't crucial.

  - Best for tasks focused purely on restoring movement without resistance training.

- Current Control Mode (For Force-Training):

  - Ideal for resistance-focused exercises.

Avoid:

- Velocity Control Mode (no position/force control).

- PWM Control Mode (complex and unnecessary for rehab)

### 1. Current Control Mode

Purpose: Directly controls the current (torque) applied to the motor.

#### Pros:

- Precise control over torque, which is essential for applications requiring variable resistance (e.g., adaptive exercises).

- Safer for rehab, as the force applied can be fine-tuned to prevent injury.

- Ideal for force-sensitive tasks.

#### Cons:

- Does not inherently control position or speed, requiring additional logic to handle motion limits.

- More complex to implement due to feedback loops.

Best For:

- Force-based rehabilitation exercises, such as resistance training.

### 2. Velocity Control Mode

Purpose: Directly controls motor speed (RPM).

#### Pros:

- Useful for repetitive motion exercises at a constant speed.

- Simple to implement for tasks requiring steady, controlled motion.

#### Cons:

- No control over the endpoint position; risk of exceeding ROM if not carefully monitored.

- Limited utility in rehab unless combined with position feedback.

Best For:

- Continuous, rhythmic motion (e.g., cyclic wrist flexion-extension at a constant speed).

### 3. Position Control Mode (0 ~ 360°)

Purpose: Moves the motor to a specific angle within a single turn.

#### Pros:

- Precise control over the endpoint of motion.

- Easy to implement for basic rehab exercises that don’t require large ranges of motion.

#### Cons:

- Limited to 360°, restricting its use for exercises requiring a broader range.

- No direct control over torque, making it less suitable for force-adaptive applications.

Best For:

- Simple, small-ROM exercises (e.g., isolated wrist movements).

### 4. Extended Position Control Mode (Multi-turn)

Purpose: Controls motor position over multiple turns, enabling a broader range of motion.

#### Pros:

- Handles large ROM exercises without constraints.

- Suitable for complex movement patterns requiring multiple turns.

#### Cons:

- Not inherently adaptive to force/resistance.

- Requires additional safety measures to prevent over-extension.

Best For:

- Large-ROM exercises or tasks requiring extensive movement.

### 5. Current-based Position Control Mode

Purpose: Combines position control with current (torque) feedback for adaptive force control.

#### Pros:

- Balances precise position control and force adaptability.

- Allows smooth, safe motion for patients with varying resistance levels.

- Ideal for rehab where resistance increases as the range expands (e.g., progressive exercises).

#### Cons:

- More complex to configure and tune than simpler modes.

- Requires a robust feedback loop and high-accuracy sensors.

Best For:

- Versatile rehab tasks that require both position control and adaptive force (e.g., progressive ROM exercises with resistance).

### 6. PWM Control Mode (Voltage Control Mode)

Purpose: Directly controls the motor by setting the applied voltage.

#### Pros:

- Direct and low-level control over the motor.

- Allows custom control strategies to be implemented from scratch.

#### Cons:

- Requires significant effort to implement advanced controls like position or velocity.

- Prone to instability and errors in high-precision tasks.

Best For:

- Custom low-level motor control (not typically suitable for rehab).